System Design Document

For

RF Direction Detection

Team members:

Abigail Butka

Cassandra Harrison

Sofia Mvokany

Kyle Reagan

Sabrina Yepez

| Version/Author | Date |
| --- | --- |
| 2.1.1/Sabrina Yepez | 02/16/2021 |
| 2.1.2/All | 02/17/2021 |
| 2.1/All | 02/18/2021 |
| 2.2.1/Sabrina Yepez | 03/17/2021 |
| 2.2/All | 03/18/2021 |

# TABLE OF Contents

1	INTRODUCTION	3

1.1	Purpose and Scope	3

1.2	Project Executive Summary	3

1.2.1	System Overview	3

1.2.2	Design Constraints	3

1.2.3	Future Contingencies	4

1.3	Document Organization	4

1.4	Project References	4

1.5	Glossary	4

2	SYSTEM ARCHITECTURE	5

2.1	System Hardware Architecture	5

2.2	System Software Architecture	5

2.3	Internal Communications Architecture	6

3	HUMAN-MACHINE INTERFACE	6

3.1	Inputs	6

3.1.2      Arduino & Stepping Motor	7

3.2	Outputs	7

3.2.1      Mission Planner	7

4	DETAILED DESIGN	10

4.1	Hardware Detailed Design	10

4.1.1 Rotating Platform & Stepping Motor	10

4.1.2 Loop Antenna	13

4.2	Software Detailed Design	15

4.3	Internal Communications Detailed Design	15

5	EXTERNAL INTERFACES	15

6	SYSTEM INTEGRITY CONTROLS	16

# System Design Document

# INTRODUCTION

## Purpose and Scope

This document describes the system requirements, operating environment, system and subsystem architecture, files and database design, input formats, output layouts, human-machine interfaces, detailed design, processing logic, and external interfaces for the RF Direction of Arrival system. The system is essentially a rotating loop antenna functioning in the 33-cm RF band that will determine the Direction of Arrival (DoA) of a simulated drone. The antenna feedback will be displayed in real-time through a computer screen.

## Project Executive Summary

This section provides a management perspective of the RF Direction of Arrival System conceptual system design framework.

### System Overview

Figure 1: System Architecture

The RF Direction of Arrival System is composed of four main sections: a stepper motor, a rotating platform, a loop antenna, and RTL-SDR receivers.

- The stepper motor will be powered by an external power supply. Its main function is to rotate 180° with the antenna platform mounted on top.

- The rotating platform is mounted on top of the stepping motor and will hold the loop antenna in place. This will allow the loop antenna to rotate with the stepper motor.

- The loop antenna is a circular 50Ω coaxial cable with a circumference of 2.7541 cm. The antenna will have a center frequency at 915 MHz but will also function in the entire 33-cm RF band (902-928 MHz). This loop antenna will send its receiving signals to the RTL-SDR .

- The RTL-SDR is a USB dongle plug-in to SMA connector. It is meant to receive antenna signals and display those signals on the SDR Sharp Airspy software. The RTL-SDR will be plugged into an external laptop, and the SDR software will also run on the same laptop.

### Design Constraints

To increase productivity and decrease design/testing cost, the system will utilize a drone without the frame or motors. It will consist of a Pixhawk flight controller, GPS, and radio transmitter. Mission planner, an external software, will be used to pair the transmitting and receiving radios to allow for real-time signal transmission. This cuts out the need to spend time creating a detailed simulation while also allowing for real-world signal transmission.

The telemetry radios are designed to operate using FHSS, which makes the signal hard to track. Decreasing the number of channels within which the signal hops and choosing a maximum and minimum frequency in the 33-cm band helps having a better view of the emitted signal.

### Future Contingencies

The telemetry radios are pre-programmed to operate on FHSS. We are currently working to mitigate the frequency hopping by reducing the number of hopped channels to 1, but there is the possibility of a contingency in the future if the code is not alterable.

The current loop antenna will be redesigned with antenna shielding and unbalanced to unbalanced connections. If a loop antenna proves to be a contingency with inconsistent results, the project may shift its focus away from loop antennas and towards dipole or Yagi antennas.

## Document Organization

This System Design Document details the system architecture in a high-level manner. It will briefly discuss the hardware and software system architecture. Along with the Human-Machine Interface where the inputs and outputs will be discussed. After that, there will be a detailed design section where it goes further into the hardware and software design aspects of the system. It will conclude with a short section discussing the integrity controls of the system.

## Project References

- ARDUINO CNC SHIELD CONTROL STEPPER. [Online]. Available: http://aconcaguasci.blogspot.com/2016/11/arduino-cnc-shield-control-stepper.html. [Accessed: 21-Sept-2020 through 24-Nov-2020].

- CNC ARDUINO SHIELD. [Online]. Available: https://courses.ideate.cmu.edu/16-375/f2017/text/resrc/cnc-shield.html#nema17-stepper-motor. [Accessed: 28-Oct-2020 through 24-Nov-2020].

- LECTURE 11: LOOP ANTENNAS. [PDF]. Available: http://www2.elo.utfsm.cl/~elo352/biblio/antenas/Lectura%2011.pdf. [Accessed: 11-Sept-2020 through 24-Nov-2020].

- HANDHELD DIRECTION-FINDING LOOP ANTENNA FOR RFI LOCATION. [PDF]. Available: http://www2.elo.utfsm.cl/~elo352/biblio/antenas/Lectura%2011.pdf. [Accessed: 11-Sept-2020 through 24-Nov-2020].

## Glossary

DOA -Direction of Arrival

MHz - Mega Hertz

dB- Decibel

RF- Radio Frequency

SDR - Software Defined Radio

FHSS – Frequency Hopping Spread Spectrum

CNC – Computerized Numerical Control

# SYSTEM ARCHITECTURE

## System Hardware Architecture

Refer to Figure 1, for system architecture. The following is the hardware implemented in the system:

- Pixhawk – GPS and transmitter radio connect to the Pixhawk. Pixhawk tells transmitter what data to send to the ground station, receiver radio.

- GPS

- Radios, transmitter and receiver

- Stepper motors - Stepper motors are connected to Arduino board for programming and power and connected to 3D-printed platform holding the antenna.

- Antenna - Antenna is connected to the stepper motor via 3D-printed platform with a space to feed cable through to connect to SDR.

## System Software Architecture

Rotating Platform

Figure 3: Class diagram of the rotating platform

The rotating platform allows the loop antenna attached to it to rotate at the speed and directions set by the user. The simple code to run the stepper motor is explained in the diagram below. Note that this is not a traditional class diagram as used in software development, since the code is so simple.

- Speed: Controls the speed of the rod attached to the stepping motor and returns the angle at which it is currently at.

- Setup: Assigns the X, Y, Z directions to the corresponding pins on the CNC and Arduino Uno boards.

- Step: The main function that outputs the angle on the serial port and transmits the speed and direction to the stepping motor.

## Internal Communications Architecture

Figure 4: Internal Communications Architecture

The loop antenna will function in the 33-cm frequency range with a peak frequency at 915MHz. This antenna will be gathering signals transmitted by Mission Planner and sending it to the RTL-SDR module via 50 ohm coaxial cable. From the RTL-SDR module, an external laptop will be connected via one of its USB ports. The laptop will then display the signals as its being recieved by the RTL-SDR software.

# HUMAN-MACHINE INTERFACE

## Inputs

- 3.1.1      Mission Planner

To perform the simulation of the drone the operator Pixhawk and Mission Planner are utilized. To do this, the Pixhawk device is attached to a telemetry radio and connected to the computer running Mission Planner. Once in Mission Planner, in the upper right-hand corner of the interface allows for customization of the type of connected device before connection. To establish the connection, the connect button shown in Figure 5 Below is used.

Figure 5: Customization of the type of connected device before connection

Once the connection is secure, the upper-right hand corner of the interface part of the interface will appear as the figure below.

Figure 6: Customization of the type of connected device after connection

### 3.1.2      Arduino & Stepping Motor

To turn the system on, plug in the Arduino board that is connected to the stepping motor and the 12-volt battery supply, to the Laptop, and upload the preexisting code. This will start the stepping motor and begin to rotate the loop antenna that is connected to the rotating platform.

## Outputs

## 3.2.1      Mission Planner

When running a Mission Planner, the user will be presented with a simulation window. This simulation will display a simulation of the virtual done including flight parameters and projected flight path. The flight parameters given include altitude, ground speed, distance to WP, yaw, vertical speed, and the distance to MAV on the first tab of parameters. Additional information about the flight and flight path can be found both on the additional tabs in the bottom right part of the interface, and within the simulated flight view in the top right part of the interface. The user with the second Pixhawk setup can see the changes in the drone’s screen in real time.

Figure 7: Mission Planner

3.2.2      Airspy

When running Airspy, the user will be presented with a graphical representation of the signals emitted by the transmitter and exchanged with the ground station. Using the loop antenna connected to the RTL-SDR dongle, the user will be able to see, in real time, the signal’s fluctuation in the chosen frequency band, here the 33 cm band expanding between 902 and 928 MHz.  The interface of Airspy helps the user play and stop the signal’s emission, change the speed of the emission, isolate the concerned bandwidth and when applicable, listen and filter noise out of to the audio radio signals exchanged on the chosen frequency.

Figure 8: Airspy display of telemetry radios communications between 914.75Hz and 915.5MHz

3.2.3      Simulink

While the signal from the telemetry radios is displayed by Airspy, similarly, Simulink is processing the signal received from the RTL-SDR. Simulink processes the data received by the RTL-SDR and converts the binary data into magnitude to display to the user. The point where the magnitude of the signal received by the antenna is the highest shows the direction from which the signal is coming. Coupled with Arduino, Simulink will display to the user, both the highest magnitude and angle at which it is achieved, to the user.

The following Simulink code is the most recent version of the signal processing. It currently outputs the raw signal without the DC component shifting it up. As more testing is done it may prove to be more beneficial to apply a complex to magnitude conversion directly onto the raw data rather than apply it after filtering. The code is not completed yet and will need to have a section of code added to interpret the raw data.

Figure 9: Preliminary Simulink Code for Detecting Telemetry Radio Communications

# DETAILED DESIGN

## Hardware Detailed Design

This section aims to depict the RF Direction Detection physical system functionality through detailed design models of the rotating platform/stepping motor, loop antenna, and the SDR. This will clarify the connections between all three systems.

### 4.1.1 Rotating Platform & Stepping Motor

The stepping motor consists of the CNC shield attached to the Arduino Uno. The motor is wired to the CNC shield and with a 12-volt battery supply. Attached to the rod of the motor is the rotating platform. The rotating platform is 3D modeled and printed. The following figures describe the schematics, wiring, and dimensions of the stepping motor and rotating platform.

Figure 10: Pin mapping and wiring of the CNC shield

Figure 11: Pin mapping and wiring of the Rotating Platform on Arduino Uno Board

Figure 12: Datasheet of the Nema 17 Stepper Motor connected to the CNC shield

Figure 13: AutoCAD drawing of the rotating platform

### 4.1.2 Loop Antenna

Figure 14: Loop Antenna HFFS simulation

The loop antenna will be approximately 32.7 mm in circumference at 915 MHz, 10% of the target wavelength, in order to achieve the simulated pattern shown in Figure 1. To make a magnetic/electrically small loop antenna, the largest linear dimension (diameter) must be 1/10 of the wavelength. The antenna must be made out of a conductive material, in this case, copper. The loop is created by stripping a coax cable’s center conductor by the desired length of the loop. It is then curved in a loop and the center conductor is then soldered onto the grounded shielding. When the incoming signal is perpendicular to the plane of the loop, the received signal gain will display a sharp drop

4.1.3 RTL-SDR

The RTL-SDR is a USB dongle that works with the Airspy software. It is composed of one USB port to connect it to the laptop and one female SMA connector to connect an antenna.

Figure 15: RTL-SDR (silver) USB dongle connected to laptop with test diamond antenna

4.1.4 GPS

The PixHawk GPS module is connected to the Pixhawk by a 4-wire cable.

Figure 16: PixHawk GPS

## Software Detailed Design

This section is not applicable as there is no major software component of the system. The Rotating platform basic software to control speed and directions and printing the angle of the loop has been described in the System Software Architecture section of the SDD

## Internal Communications Detailed Design

The RTL-SDR transcribes the signal received by the loop antenna into a visual representation of the signal’s spectrum, displaying the gain in dB and the frequency in MHz. This signal is emitted by the telemetry radios, transmitting and receiving flight pattern’s information from the PixHawk flight controller.

# EXTERNAL INTERFACES

The only other systems that work alongside the RF Directions Detection system will be any drone within the area that is in the detection range of the system.

5.1      Interface Architecture

The communication between the drone in question is described in the Human-Machine Interfaces Input section of the SDD. The frequency the drone runs on will be scanned by the loop antenna and the software-defined radio. Once detected by the system the Rotating Platform will stop so the loop antenna and software design radio can continue receiving the signal of the drone and display

# SYSTEM INTEGRITY CONTROLS

There is no sensitive information to the software or hardware of the system. The system will be available to Embry-Riddle students as a demonstration of RF Direction Detection.